## Practical 7

## Lab Practical: Application Layer Packet Analysis using Packet Tracer Tool

### Aim:

To analyze Application Layer packets (DNS & HTTP) using a packet tracer tool (Wireshark) and understand their structure and working.

### Tools Required:

- Wireshark (Packet Analyzer)

- Internet Connection or Local DNS/HTTP Server

- Browser or Command Line Tools (nslookup, ping)

### Procedure:

#### Part A: DNS Packet Analysis

- Start Wireshark and begin packet capture on the active network interface (WiFi/Ethernet).

- Open Command Prompt or Terminal and type:

- nslookup www.google.com

- In Wireshark, stop the capture after response.

- Apply filter:

- dns

- Select the DNS query and observe the following:

  - Transaction ID

  - Flags: Query/Response

  - Query Name: e.g., www.google.com

  - Query Type: A (IPv4)

- Click on the DNS Response packet and observe:

  - Resolved IP Address

  - TTL (Time to Live)

  - Answer Section

#### Part B: HTTP Packet Analysis

- In the browser, type:

- http://example.com

- In Wireshark, apply filter:

- http

- Observe the HTTP GET request:

  - Method: GET

  - Host: example.com

  - User-Agent

- Observe the HTTP Response:

  - Status Code: 200 OK

  - Content-Type: text/html

  - Content-Length

### Observations:

| Field | DNS Query | DNS Response | HTTP Request | HTTP Response |
| --- | --- | --- | --- | --- |
| Protocol | UDP | UDP | TCP | TCP |
| Port | 53 | 53 | 80 | 80 |
| Method/Type | Standard Query (A Record) | Response | GET / | 200 OK |
| IP Address Queried | www.google.com | e.g., 142.250.77.68 | example.com | IP of example.com |
| TTL | - | 300 seconds (example) | - | - |
| Content-Type | - | - | - | text/html |

### Result:

Successfully analyzed DNS and HTTP packets using Wireshark. The structure and fields of Application Layer packets were observed and interpreted.

### Conclusion:

- DNS resolves domain names to IP addresses via UDP port 53.

- HTTP works over TCP port 80, where the client sends a GET request and receives a response with status and content.

- Wireshark effectively captures and visualizes Application Layer communication for analysis and troubleshooting.